Sensitivity analysis of parameters

The evaluation is conducted on the Street06 and Street08 in the public M2DGR dataset.

(1) Sensitivity analysis of t

t was varied within {0.45, 0.50, 0.55, 0.60, 0.65, 0.70, 0.75} while all other parameters were fixed at their default values.

Tab. 2 Metrics for t sensitivity analysis on the Street06 sequence.

| t | RMSE (m) | Δ |
| --- | --- | --- |
| 0.45 | 0.45 | +7.1% |
| 0.50 | 0.43 | +2.4% |
| 0.55 | 0.42 | +0.0% |
| 0.60 | 0.42 | — |
| 0.65 | 0.44 | +4.8% |
| 0.70 | 0.50 | +19.0% |
| 0.75 | 0.57 | +35.7% |

Tab.3 Metrics for t sensitivity analysis on the Street08 sequence.

| t | RMSE (m) | Δ |
| --- | --- | --- |
| 0.45 | 0.49 | +6.5% |
| 0.50 | 0.47 | +2.2% |
| 0.55 | 0.46 | +0.0% |
| 0.60 | 0.46 | — |
| 0.65 | 0.48 | +4.3% |
| 0.70 | 0.54 | +17.4% |
| 0.75 | 0.61 | +32.6% |

When t < 0.60, the relaxed truncation allows a higher density ratio to enter the Sigmoid, causing α to increase. Even under moderate field of view conditions, the system still overreacts due to more refined downsampling and more iterations, resulting in unnecessary computations and only a slight decrease in RMSE.

When t > 0.60, the stricter truncation will suppress the density ratio, thereby reducing α. This adaptive enhancement method is weakened when the narrow field of view geometry requires a larger α to achieve stronger compensation.

t ∈ [0.50, 0.65] maintains RMSE within 5% of the optimum on both sequences.

(2) Analysis of Center Offset s

s was varied within {0.40, 0.45, 0.50, 0.55, 0.60, 0.65, 0.70} while all other parameters were fixed at their default values. The evaluation is conducted on the M2DGR Street06 and Street08 sequences.

Tab. 4 Metrics for sensitivity analysis of s on the Street06 sequence.

| s | RMSE (m) | Δ |
| --- | --- | --- |
| 0.40 | 0.47 | +11.9% |
| 0.45 | 0.44 | +4.8% |
| 0.50 | 0.43 | +2.4% |
| 0.55 | 0.42 | — |
| 0.60 | 0.43 | +2.4% |
| 0.65 | 0.45 | +7.1% |
| 0.70 | 0.48 | +14.3% |

Tab. 5 Metrics for sensitivity analysis of s on the Street08 sequence.

| s | RMSE (m) | Δ |
| --- | --- | --- |
| 0.40 | 0.51 | +10.9% |
| 0.45 | 0.48 | +4.3% |
| 0.50 | 0.47 | +2.2% |
| 0.55 | 0.46 | — |
| 0.60 | 0.47 | +2.2% |
| 0.65 | 0.49 | +6.5% |
| 0.70 | 0.52 | +13.0% |

When s = 0.40, the midpoint position is below the typical density ratio range of the urban scenario. Therefore, even under slight field of view limitations, α still tends to approach 1, resulting in an increase in computational cost without an improvement in accuracy.

When s = 0.70, the midpoint exceeds the density ratio range typically encountered, causing α to remain close to 0, thus unable to promptly compensate for the impact of field of view changes.

The robust interval s ∈ [0.50, 0.60] enables α to be well calibrated in most typical scenarios of the urban environment, with the RMSE controlled within +2.4%.

(3) Analysis of Steepness k

k was varied within {4.0, 5.0, 6.0, 7.0, 8.0, 9.0, 10.0, 12.0} while all other parameters were fixed at their default values. The evaluation is conducted on the M2DGR Street06 and Street08 sequences.

Tab. 6 Metrics for sensitivity analysis of k on the Street06 sequence.

| k | RMSE (m) | Δ |
| --- | --- | --- |
| 4.0 | 0.48 | +14.3% |
| 5.0 | 0.45 | +7.1% |
| 6.0 | 0.44 | +4.8% |
| 7.0 | 0.43 | +2.4% |
| 8.0 | 0.42 | — |
| 9.0 | 0.43 | +2.4% |
| 10.0 | 0.46 | +9.5% |
| 12.0 | 0.51 | +21.4% |

Tab. 7 Metrics for sensitivity analysis of k on the Street08 sequence.

| k | RMSE (m) | Δ |
| --- | --- | --- |
| 4.0 | 0.52 | +13.0% |
| 5.0 | 0.49 | +6.5% |
| 6.0 | 0.48 | +4.3% |
| 7.0 | 0.47 | +2.2% |
| 8.0 | 0.46 | — |
| 9.0 | 0.47 | +2.2% |
| 10.0 | 0.50 | +8.7% |
| 12.0 | 0.55 | +19.6% |

When k = 4.0, the asymptotic Sigmoid curve disables the adaptive distinction between different field of view states.

When k = 12.0, the steep transition causes α to suddenly jump between frames as the density ratio crosses the midpoint, introducing temporal oscillations in the downsampling resolution and map density.

The range k ∈ [7.0, 8.5] can provide a balanced α response curve, keeping the RMSE within +2.4%.

(4) Combined sensitivity analysis of spatial radii Rinner and Router

All combinations of Rinner ∈ {3, 4, 5, 6, 7} m and Router ∈ {12, 15, 20, 22, 25} m were evaluated on the M2DGR Street06 and Street08 sequences, with all other parameters fixed at defaults.

Tab. 8 RMSE (m) for sensitivity analysis of Rinner and Router on the Street06.

| Router Rinner | 12 m | 15 m | 20 m | 22 m | 25 m |
| --- | --- | --- | --- | --- | --- |
| 3 m | 0.55 (+31.0%) | 0.52 (+23.8%) | 0.49 (+16.7%) | 0.50 (+19.0%) | 0.52 (+23.8%) |
| 4 m | 0.50 (+19.0%) | 0.47 (+11.9%) | 0.44 (+4.8%) | 0.45 (+7.1%) | 0.47 (+11.9%) |
| 5 m | 0.47 (+11.9%) | 0.44 (+4.8%) | 0.42 (—) | 0.43 (+2.4%) | 0.45 (+7.1%) |
| 6 m | 0.51 (+21.4%) | 0.47 (+11.9%) | 0.44 (+4.8%) | 0.45 (+7.1%) | 0.47 (+11.9%) |
| 7 m | 0.56 (+33.3%) | 0.52 (+23.8%) | 0.48 (+14.3%) | 0.50 (+19.0%) | 0.53 (+26.2%) |

Tab. 9 RMSE (m) for sensitivity analysis of Rinner and Router on the Street08.

| Router Rinner | 12 m | 15 m | 20 m | 22 m | 25 m |
| --- | --- | --- | --- | --- | --- |
| 3 m | 0.60 (+30.4%) | 0.57 (+23.9%) | 0.54 (+17.4%) | 0.55 (+19.6%) | 0.57 (+23.9%) |
| 4 m | 0.54 (+17.4%) | 0.51 (+10.9%) | 0.48 (+4.3%) | 0.49 (+6.5%) | 0.51 (+10.9%) |
| 5 m | 0.51 (+10.9%) | 0.48 (+4.3%) | 0.46 (—) | 0.47 (+2.2%) | 0.49 (+6.5%) |
| 6 m | 0.55 (+19.6%) | 0.51 (+10.9%) | 0.48 (+4.3%) | 0.49 (+6.5%) | 0.51 (+10.9%) |
| 7 m | 0.61 (+32.6%) | 0.57 (+23.9%) | 0.52 (+13.0%) | 0.54 (+17.4%) | 0.57 (+23.9%) |

At Rinner = 3 m, too few near-field points yield a density ratio dominated by sampling noise rather than scene structure, increasing RMSE by up to 33.3%. When Rinner = 7 m, the scale difference between the local details and the overall structure is not reflected.

At Router = 12 m, the outer shell fails to span typical urban street widths, contaminating the background density reference with local fluctuations. At Router = 25 m, distant sparse returns from building edges and foliage destabilize the density ratio.

The robust region Rinner ∈ [4, 6] m, Router ∈ [15, 22] m maintains RMSE within 11.9% across both sequences.

(5) Summary of sensitivity results

To provide a consolidated view, the sensitivity ranges for all five parameters is summarized based on the M2DGR Street06 and Street08 sequences:

Tab. 10 Summary of Sensitivity Results

| Parameter | Default | Robust Range | RMSE Variation in Robust Range |
| --- | --- | --- | --- |
| Rinner | 5 m | 4–6 m | +2.4% to +4.8% |
| Router | 20 m | 15–22 m | +2.4% to +7.1% |
| t | 0.60 | 0.50–0.65 | +0.0% to +4.8% |
| s | 0.55 | 0.50–0.60 | +0.0% to +2.4% |
| k | 8.0 | 7.0–8.5 | +0.0% to +2.4% |